https://sam.gov/workspace/contract/opp/279b68b7601646eba64a20c7d9ffa952/view

Duct Cleaning

Active

Opportunity

Notice ID

12444026Q0119

Related Notice

(blank)

Contract Opportunity Type

Combined Synopsis/Solicitation

Contract Line Item Number

(blank)

Inactive Dates

Jul 13, 2026

Inactive Policy

15 days after date offers due

Date Offers Due

Jun 28, 2026 9:00 AM EDT

Published Date

Jun 18, 2026 4:32 PM EDT

Department/Ind. Agency

AGRICULTURE, DEPARTMENT OF

Sub-tier

FOREST SERVICE

Office

USDA-FS, CSA EAST 2

Classification

Original Set Aside

Total Small Business Set-Aside (FAR 19.5)

Product Service Code

4120 - AIR CONDITIONING EQUIPMENT

NAICS Code

333415 - Air-Conditioning and Warm Air Heating Equipment and Commercial and Industrial Refrigeration Equipment Manufacturing

Place of Performance

(blank)

Initiative

None

Description

*see attachments

*solicitation end date changed to 06-28-2026 by 0900hrs AM ET to allow for more competition to this requirement.

STATEMENT OF OBJECTIVES (SOO)

Summary of Work:  Cleaning of HVAC ductwork in the U.S. Forest Service Supervisor Office, located at 605 West Main St. Russellville, AR. 72801.

The contractor will provide all labor, tools, equipment, and materials to perform the following work on 12 HVAC units.

Remove, clean, and reinstall all supply and return registers.

Place ductwork in negative air pressure with HEPA filter negative air vacuum.

Clean critical HVAC unit components such as blower motors, evaporator coils and plenum.

Mechanically scrub and/or air whip all supply and return ductwork.

Treat any supply or return ductwork with signs of mold with Sporicidin antifungal disinfectant or approved equal.

PROJECT COORDINATION

Notify Contracting Officers Representative (COR) with the USDA Forest Service, 5 days prior to start of work. Timing of the work will be coordinated with the COR to minimize disruption to daily operations. Possible weekend cleaning will be needed. The contractor will perform the work in a manner that will protect the safety of public and employees. Access to the building by F.S. personnel may be necessary during cleaning activities. Necessary steps will be taken to ensure the safety and preservation of all Forest Service property inside the building.

Contractor will follow the standards for HVAC system cleaning set by the National Air Duct Cleaners Association (NADCA). Certification by the NADCA is recommended.

Contact Information

Primary Point of Contact

Danny Sims

Email

danny.sims@usda.gov

Phone Number

479-223-2176

Alternative Point of Contact

Richard Kandrach

Email

richard.kandrach@usda.gov

Phone Number

000-000-0000

Contracting Office Address

1720 Peachtree ST NW STE 876S

(No Street Address 2)

Atlanta, GA 303092449 USA

Attachments/Links

Links

No links have been added to this opportunity.

Attachments

Download All

Request Access

Document	File Size	Access	Updated Date

C04_Solicitation Q A v1_12444026Q0119.pdf

90.4 KB	 Public	Jun 19, 2026

C04_00 Solicitation Template_12444026Q0119.pdf

416.2 KB	 Public	Jun 19, 2026

C04_01 SOW_12444026Q0119.pdf

602.33 KB	 Public	Jun 18, 2026

C04_02 WD Pope County AR_12444026Q0119.pdf

4.17 MB	 Public	Jun 18, 2026