CITY OF RICHMOND

DEPARTMENT OF PUBLIC WORKS

SPECIAL PROVISIONS

US 60 Downtown Expressway Gateway Pedestrian Improvements (UPC#111702)

- GENERAL - The basic specifications for this project shall be the 2020 issue of the Virginia Department of Transportation Road and Bridge Specifications http://virginiadot.org/business/const/spec-default.asp, except for those Sections that may be in conflict with the City of Richmond Standards or Contract documents, in which case the City Standards or Contract documents shall govern; and wherein the above specifications, the word "State" appears, the word "City" is to be substituted and where the word "Department" appears, it shall mean the Department of Public Works within the City of Richmond.  In the event, any city, state or federal funds are utilized, the appropriate city, state or federal guidelines must be adhered to.

These special provisions are intended to add to and more fully describe the requirements not otherwise detailed or set forth on the plans and specifications.  In addition, when necessary, the provisions are to amplify or modify the character of workmanship and materials.

Copies of all forms, releases, standards, drawings, specifications, handbooks, orders, and codes referred to in this proposal are available in the offices of the Department of Public Works within the City of Richmond, Virginia.

- STANDARDS & REVISIONS TO THE GENERAL CONDITIONS OF THE CONTRACT - In addition to the requirements stated in the Virginia Department of Transportation Road and Bridge Specifications, the following requirements supplement, add to or modify those Specifications and shall govern in all conflicting cases.

Applicable Standards - The work shall be done in accordance with the latest editions of the following, in effect on the date of the invitation to bid:

- City of Richmond, Department of Public Works “Right-of-Way Roadway Geometric Design Manual.”

- City of Richmond, Department of Public Works “Right of Way Excavation & Restoration Manual.” https://www.rva.gov/sites/default/files/2019-08/Excavation_Restoration_Manual.pdf

- City of Richmond Department of Public Works – A policy for Temporary Steel Plate Bridging for General Utility Street Work. https://www.rva.gov/sites/default/files/2019-08/Excavation_Restoration_Manual.pdf

- City of Richmond Department of Public Works – Traffic Order #2844, dated April 2011, Traffic Control Specifications for Work in Street and Alley Permits.  https://www.rva.gov/sites/default/files/2019-08/Excavation_Restoration_Manual.pdf

- City of Richmond, Department of Public Utilities “Stormwater Management Design and Construction Standards Manual.” https://www.rva.gov/sites/default/files/2021-06/Stormwater_Management_Design_and_Construction_Standards_Manual.pdf

- City of Richmond, Department of Public Utilities “Sanitary Sewer System Design Guidelines and Standard Specifications and Details” Manual. https://rva.gov/sites/default/files/2022-04/SANITARY%20SEWER%20DESIGN%20GUIDELINES%20%26%20STANDARDS%20-%20MARCH%202022%20-%20PROPOSED%20%28final%29.pdf

- City of Richmond, Department of Public Utilities “Water Distribution System Design Guidelines and Standard Specifications and Details” Manual. https://www.rva.gov/sites/default/files/2019-03/waterdistributionguidelines_specifications.pdf

- City of Richmond, Department of Public Utilities “Cross Connection Control” Manual. https://www.rva.gov/sites/default/files/2019-03/Cross_Connection_Manual.pdf

- City of Richmond Standard Specifications for Gas, Water System, Construction and Maintenance DPU No. 1-92.

- Virginia Department of Transportation 2020 Road and Bridge Specifications https://www.virginiadot.org/business/resources/const/VDOT_2020_RB_Specs.pdf

- Virginia Department of Transportation Work Area Protection Manual, latest revision. http://www.virginiadot.org/business/trafficeng-wzs.asp

- Virginia Department of Transportation 2016 Road and Bridge Standards http://www.virginiadot.org/business/locdes/2016_road_and_bridge_standards.asp

- Latest rules and regulations governing construction, demolition and excavation as adopted by the Safety Codes Commission of the Commonwealth of Virginia. https://www.doli.virginia.gov/

- Virginia Erosion and Sediment Control [E&SC] Handbook, Virginia Department of Environmental Quality. https://www.deq.virginia.gov/water/stormwater/esc-handbook

- Federal Highway Administration, USDOT, Manual on Uniform Traffic Control Devices, latest revision.  https://mutcd.fhwa.dot.gov/pdfs/11th_Edition/mutcd11thedition.pdf

- OSHA Regulations (Standards – 29 CFR). https://www.osha.gov/

- Sound Control Ordinance of the City of Richmond https://library.municode.com/va/richmond/codes/code_of_ordinances?nodeId=PTIICICO_CH11EN_ARTIISOCO

- Bird Netting Requirements.

- ADDITIONAL REQUIREMENTS –

By submitting a bid for this project the contractor certifies that they are familiar with the requirements of all the contract documents and that all subcontractors are familiar with the requirements of the contract documents as they may be additionally affected.

- OWNER REPRESENTATION - The work shall be done under the general supervision of the Director of Public Works with direct supervision by the Capital Projects Manager, who shall assign a Project Manager engineer and a Project Inspector at the pre-construction meeting to the project.  The presence or absence of the Owner's representative shall in no way relieve the contractor of  their responsibility to complete the work in accordance with the Contract Documents or lessen his obligation to correct unsatisfactory work.  All required submittals shall be submitted to the Project Manager through the Project Inspector.

- PRE-CONSTRUCTION MEETING - After award of the contract and prior to commencement of the work, a meeting will be arranged between the contractor, Purchasing Officer, Project Manager, Project Inspector, and Construction Manager to discuss all aspects of project, including Public Works Department policies and regulations, administrative matters, safety and accident prevention, and will also provide the contractor an opportunity to request clarification of any of the contract documents and Owner requirements.

- COORDINATION- The Contractor shall coordinate work within the project area, to include any detour routes, with other contractors or work by City forces within the same geographic region in accordance with VDOT Specifications Section 105.09.

- SCHEDULING - The Contractor  shall submit a written schedule as required by the contract documents.

- TESTING-  A minimum of three business day notice shall be given to the Inspector prior to performing any activity that requires Quality Control Testing. The Contractor is to avoid performing any activities that require Quality Control testing in the late afternoon. Should the Contractor choose to proceed in executing work without inspection, they do so at their own risk and may be required to remove such work without charge at the discretion of the Project Manager.

- SEQUENCE OF WORK- On projects involving more than one street or block, the work shall be so scheduled as to provide a continuous and uninterrupted flow of all work items in a logical sequence in each street or block. If, in the opinion of the Owner, the flow of all work items is not being followed in accordance with the approved schedule on a block-by-block and street-by-street basis, the Owner may suspend work on all or  portions thereof until such time as those items not being prosecuted sequentially are completed or on schedule.  Additional time will not be allowed for the remaining work for the time the suspension was in effect.

- HOLIDAYS - The Contractor shall not permit work, except that which is necessary to maintain traffic, to be done on Sundays and  city observed holidays without written consent of the Owner.

- EXISTING INFRASTRUCTURE - The location of existing sewer, water or gas lines, conduits, underground cables or other structures across or along the line of the proposed work may vary from the locations shown on the drawings and, where shown, the locations depth and dimensions of such structures are approximately correct and may vary.  The Contractor shall be responsible for determining the exact location of the lines or structures and will be liable for repairs in the event of damage.

  - The Contractor's shall notify and coordinate with "Miss Utility" (811) prior to the beginning of construction.

  - The Contractor shall notify the proper City agencies and coordinate with all private utilities at least 3 business days prior to the time set for work involving their facilities or equipment.  Furthermore, this responsibility shall extend to coordinating all contract work with existing utility owners to avert or minimize any conflicts whether apparent or unforeseen.  The owner or their representative may allow a contract time extension, at their sole discretion, based on full documentation received from the contractor.

    - Department of Public Utilities:  The Contractor shall arrange for adjustment of fire hydrants, gas, water or electrical lines and such work shall be performed by contractors prequalified by the Department of Public Utilities.  Documentation of pre-qualification by DPU shall be provided to the Project Inspector at least 5 business days prior to commencement of utility work. If the actual adjustment of fire hydrants, gas, water or electrical lines are made by City personnel, the Contractor's responsibility shall consist of uncovering and properly restoring the utility.

    - Traffic Signal Equipment:  A representative from the Division of Traffic Engineering must be present before beginning any work involving their equipment and/or excavation in the vicinity of signalized intersections. Traffic loops damaged during excavation shall be repaired at the contractor’s expense. Installation and relocation of this equipment will be by others unless included as a bid item.  The Contractor shall cooperate with others for this work with no additional time allowed.

## When installations or work is required at congested traffic intersections, as defined by the City of Richmond, it is required that at least one of the successful contractor(s) and/or subcontractor(s) personnel on site shall be:

## •    IMSA Level II certified
•    VDOT Intermediate Work Zone Safety Training certified

## At locations where work is called for in manholes, follow all applicable City, State, and Federal (OSHA) requirements regarding work in enclosed spaces. Work within the City’s Department of Public Works (DPW) Streetlights Division and Transportation Engineering

## Division joint-use manholes requires contractors to have certified personnel for cabling installation in a medium voltage (2400 through 15000 volts) work environment and be certified to work in enclosed spaces.

## When working with fiber optic splicing, the technician performing the work shall have a Fiber Optic Installer (FOI) certification.

- OFF DUTY UNIFORMED POLICE ‑ During the course of the work it may be necessary to employ off‑duty uniformed police for traffic control.  They shall be employed only when approved by the Project Manager.  The Contractor shall pay police officers $ 56.00 per hour and Supervisors $ 61.00 per hour if they report for work.  Whenever three or more police officers are employed off-duty, at least one must function as a Supervisor.  Off-duty police shall be paid at time-and-a-half when working on City of Richmond holidays and New Year's Eve.  The contractor shall pay the Richmond Police Department $ 35.00 per day for usage per each marked police car.  The City will reimburse the Contractor at the rate plus $ 0.50 per hour for police officers (regardless of rank) and use of marked police car, minimum payment of four hours for officers reporting for duty.  Reimbursement shall not be made when officers report for work, the Contractor does not work and does not request cancellation of officers.  When requesting Police Officers, the contractor shall contact the Police Department (Police Outside Employment, 804-646-0455 or Adriane.James@RVA.GOV) a minimum of 5 business days prior to the anticipated reporting time for the required officers.  A copy of the request for use of uniformed police officers shall be sent to the City’s Project Manager.

- SITE MAINTENANCE - Throughout all phases of construction, the Contractor shall keep the work site clean and free from rubbish, debris, and dust.  Dust nuisance shall be abated by cleaning, sweeping, sprinkling with water or calcium chloride at no additional cost to the City.  Care shall be taken to prevent spillage on haul routes.  Any such spillage shall be removed immediately and the route cleaned.  If the use of water results in mud tracking on adjacent streets, the project may be shut down until streets are cleaned by the Contractor at Contractor’s expense and no extra time will be allowed for the time thus lost.  Materials and equipment shall be removed from the site as soon as they are no longer needed.  If the Contractor does not respond to the requests to clean the streets, they shall be cleaned by the City and the cost shall be deducted from the payments due the Contractor.

- CONSTRUCTION STAGING AREA - The contractor shall be responsible for selecting the staging and laydown areas and any needed storage yard in the vicinity of the project. Areas used by the contractor within the public right-of-way require the prior approval of the Project Manager.  All costs associated with the use, maintenance, and final restoration of staging areas needed shall be at the sole expense of the contractor.  In addition, the Contractor shall do the following:

  - Arrange for and maintain the staging area at Contractor expense.

  - Locate trailers and equipment as far as possible from nearby occupied dwellings.

  - Keep the site neat and policed so that debris shall not be transported to neighboring properties by wind or other means.

  - Start construction equipment only when necessary in the mornings.

  - Do not leave construction equipment running needlessly.

  - Caution workmen to speak quietly and to use language that would not offend citizens in the area.

  - Locate portable sanitary facilities on a secluded or concealed portion of the site.

  - Where appropriate, provide lighting and/or fencing to make access to the site during non-working hours more difficult.

  - When storing construction materials insure that they do not have a tendency to become unstable.

  - Where dust may become a nuisance, provide means for dust control.

  - The Contractor shall maintain access to all properties and city services.

  - Grass and weeds within the project limits shall be kept trimmed at all times by the contractor at no additional cost to the city.

- SITE SAFETY- Safety is the contractor’s responsibility; all barricades, safety fencing, temporary signage, etc. are incidental to the contract. The Contractor shall also inspect the project area for unsafe conditions prior to mobilization. Should unsafe conditions be found the Contractor shall immediately contact the responsible party and shall not mobilize until sufficient repairs are made to make the area safe. The contractor may request a contract time adjustment if in  their opinion, repairs created a significant delay to their critical path activities.  Provided prior notification was timely, the Project Manager, at their sole discretion, may grant such time extension based on written documentation duly furnished by the contractor.  No delay claim will be receivable if, in the opinion of the Project Manager, the responsible party has accomplished due diligence in adjusting its facilities within 10 (ten) working days of the above notification.

- DISPOSAL OF EXCESS MATERIALS - All inert materials not designated for salvage (concrete, brick, uncontaminated soils, cobblestone, etc.) from the project shall be disposed of at an approved landfill and the cost of loading and hauling to the landfill shall be incidental to the unit price of other items.

- MOBILIZATION - Mobilization shall not be paid unless noted as a separate bid item.

- TRAFFIC CONTROL – Traffic Control will not be paid and is considered incidental to unit prices where work is to be performed in residential/local streets.  Where work is to be performed in other than residential streets, the Project Manager will either provide a maintenance of traffic plan as part of the project paying for traffic control on a unit cost basis or require the contractor to provide a lump sum quote to provide traffic control on the project based on the latest Work Area Protection Manual.

- SURVEYING – When a pay item, surveying shall be paid in accordance with the specifications and these special provisions.  If not a pay item, surveying shall be incidental to other unit prices and shall not be paid separately.  Pricing shall include full compensation for all material, equipment and labor necessary to perform the work.

- COPIES FURNISHED TO CONTRACTOR - After the contract has been executed, the Contractor will be furnished one (1) set of the Contract Drawings and Specifications in portable document format (PDF) and only one (1) full size set of drawings/sketches. It shall be the Contractor's responsibility to furnish each of their subcontractors, manufacturers and material suppliers such copies of the Contract Documents, as may be required for their respective work.

- LUMP SUM CONTRACT ITEMS – Measurement and Payment of Lump Sum items, such as Traffic Control and Construction Surveying, when pay items, will be made in installments based on work completed from the schedule of values.  The schedule of values shall not be consider lump sum contract items (Mobilization, Traffic Control, Surveying, etc.) for calculation of work progression.  Mobilization, when a pay item, will be paid on the first two payments in accordance with the specifications.

- VIRGINIA DEPARTMENT OF TRANSPORTATION ROAD & BRIDGE SPECIFICATIONS - MODIFICATIONS:  ADDITIONAL PROJECT REQUIREMENTS

In the event of a conflict between the referenced VDOT Road and Bridge Specifications and the modifications thereto listed below, the below listed modifications shall govern.

- SECTION 315 – ASPHALT CONCRETE PAVEMENT IS AMENDED AS FOLLOWS:

- Certified Flagmen or off-duty Police Officers shall be provided, as directed by the Project Manager.  The cost of the Certified Flagmen shall be paid at the unit price bid unless otherwise provided for under maintenance of traffic bid item. If Flagmen are requested by the Project Manager, they must be certified by VDOT, the American Traffic Institute, or other approved entity and if evidence of certification is not provided, the work will be suspended until proper documentation is provided to the Inspector. Additional time for stop work order due to uncertified flagman will not be allowed.

- SECTION 412 – WIDENING, REPAIRING, AND RECONSTRUCTING EXISTING STRUCTURES IS AMENDED TO INCLUDE THE FOLLOWING:

- Removal of accumulated wastes - The Contractor shall remove and disposal of any bird waste and any other debris accumulated on the structural members and bridge seats in areas where work is being performed. Bird waste and debris material contaminates may require special handling and disposal in accordance with all Federal, state, and local requirements.

## No separate payment shall be made for removal and disposal of accumulated wastes. Cost shall be incidental to the contract pay items.

- SECTION 517 – CONTRACTOR CONSTRUCTION SURVEYING IS SUPPLEMENTED AS FOLLOWS:

Measure and Payment for Construction Surveying, when a bid item, shall be at the contract unit price in accordance with these special provisions.  When not a pay item, the costs of Construction Surveying shall be included in the price bid for other appropriate items.

- ADJUSTING UTILITIES AND CASTINGS:

- Resetting Castings - The Contractor shall be required to adjust sewer manhole castings, gas, water valve and drip boxes to new grades.  For large valve and drip boxes the vertical adjustment shall be made with brick and concrete.  The entire base of the box shall be placed on brick when set to new grade.  For small valve boxes the vertical adjustment is made by screwing the top section and/or by installing a new mid-section.

- Sewer manhole castings shall be reset to grade on brick with high-early concrete or precast concrete adjustment rings.  Virginia Power and telephone manhole castings will be reset by the respective utility, unl.  Old pavement removed to adjust gas, water valve and drip boxes and sewer manhole castings shall be replaced with concrete on graded streets.  The contractor shall coordinate adjustment of private utility owned manholes with the respective owner.

- The Contractor shall properly place and fit the sewer rings and adapters.  Rings and adapters improperly placed shall be cut out and replaced at Contractor expense.

- The Contractor shall remove all materials that fall in the sewer manholes and basins and reset all castings the day the castings are disturbed.  Work shall not be resumed the next day or on any new work until such time as previous day's work is complete (i.e.:  the Contractor not cleaned out any sewer manhole or basin or reset the castings of previous day's work).

- Concrete collars around sewer manhole castings and valve boxes shall be Class "A3" Type I or II cement and shall be protected from traffic for three (3) days.  Backfill for valve boxes set to grade shall be compacted with a pneumatic tamp to a minimum of ninety-five (95%) percent density (Standard Proctor).

- Sewer castings, gas, water valve and drip boxes where the final surface shall be concrete, shall be set to grade without concrete collar.

- All castings shall be adjusted via methods such that they do not settle from the adjusted elevation.

- Adjusting Utilities - In the event that any existing gas, water lines, fire hydrants or services have to be adjusted and not shown on the plan, the Contractor shall notify the Department of Public Utilities (DPU) seventy-two (72) hours prior to the time set for uncovering the existing lines, adjusting hydrants, or adjusting service boxes.  Any adjustment to gas mains, water lines or fire hydrants, unless included in the contract, will be done by the Department of Public Utilities' forces.  Uncovering lines shall be done by the Contractor.  The contractor may request a contract time adjustment if in their opinion, DPU in adjusting its facilities created a significant delay to their critical path activities.  Provided prior notification was timely, the Project Manager, at their sole discretion, may grant such time extension based on written documentation duly furnished by the contractor.  No delay claim will be receivable if, in the opinion of the Project Manager, DPU has accomplished due diligence in adjusting its facilities within 10 (ten) working days of the above notification.  The same process shall apply to other private and public utilities with facilities within the public right-of-way.

- Payment:

- Payment for manholes set to grade shall be made at the contract unit price bid for this item and shall include the material, labor and equipment necessary to perform the work.

- Payment for adjusting gas, water valve, and drip boxes shall be made at the contract unit price bid for "Adjust utility castings, large (greater than 16 dia.)” or “Adjust Utility Casting, Small (6"-16")” and shall include the material, labor and equipment necessary to perform the work.

- Water meter boxes and gas curb cocks shall be made at the contract unit price for “Adjust water meter or gas cock to grade" and shall include the material, labor and equipment necessary to perform the work.

- SAW  CUTTING – All existing asphalt & concrete pavements shall require saw cut in order to create a neat line, no other methods will be allowed.  Should the contractor use an unacceptable method to create the pavement cut, the contractor shall establish a saw cut beyond the limits of work using acceptable methodology to create a neat joint at no additional cost to the City.

- MAINTENANCE OF TRAFFIC- the Contractor may submit an alternate Maintenance of Traffic plan as part of the requirements for the Work in Street permit.

## The following terms are used interchangeably:

- Maintenance of Traffic Plan

- Traffic Management Plan

- Transportation Management Plan

- Temporary Traffic Control

## The following procedure shall be followed for all street and sidewalk construction projects which may create obstructions to traffic movements in accordance with Traffic Order #2844, dated August 12, 1998 – Traffic Control Procedure for Street and Sidewalk Construction.

- General:

- The Transportation Engineering Division shall be notified not less than 72 hours prior to beginning street, utility or sidewalk construction. If work requires changes in bus routes, bus stops or traffic detours, a longer advance notice may be required.  Under no circumstances shall a roadway be closed for routine construction without the approval of the City Traffic Engineer or agent, and prior notification to the Division of Emergency Communications, telephone 804-646-5123.

- In case of EMERGENCY WORK during normal business hours (8:00am to 5:00pm, Monday-Friday), notify the Transportation Engineering Division, telephone 646-0442, as soon as possible. At all other times notify the Division of Emergency Communications, telephone 804-646-5123, who in turn will notify Fire and Emergency Services.

- Metal plates shall be required over street openings to maintain adequate traffic movement in order to reopen travel lanes during peak traffic periods. Metal plates and the installation thereof shall conform to DPW Policy for Temporary Steel Plate Bridging for General Utility Street Work dated April 2004.

- No vehicles shall be parked on the street or in alleys, at construction sites in violation of “No Parking” regulations unless the vehicle is essential to the completion of the job. Whenever a traffic lane is blocked by construction work, all vehicles, equipment and materials shall be parked or stored in the same lane.

- No vehicles shall be parked neither on the streets nor on the sidewalks behind fences and barricades on construction and demolition projects. The area behind fences and barricades, whether in the street or on the sidewalk area, is for either loading and unloading, or storage of necessary materials. Materials shall not be stored where they may cause a safety hazard by obstructing the vision of drivers and pedestrians.

- Any traffic signs, signals, or pavement markings removed as a result of the construction shall be replaced by the contractor. Before removing any traffic control device, the Transportation Engineering Division shall be notified in order that temporary safety arrangements can be made. Any signs removed shall be delivered to the Sign Shop, 2800 Hermitage Road by the person removing same or agent.

- All flaggers shall be certified by VDOT, the American Traffic Institute, or other approved entity. The contractor shall provide copies of certifications of flaggers to be used to the Project Inspector prior to the commencement of work.  Further, the flagger shall have the certification card on their person at all times while engaged in flagging operations.

- The Contractor shall plan and stage operations such as to maintain access to all properties, especially for emergency services, at all times.

- Traffic Control Devices:

- All traffic control devices shall be in conformance with the Federal Highway Administration’s Manual on Uniform Traffic Control Devices for Streets and Highways and the Virginia Department of Transportation’s Work Area Protection Manual.

- The contractor shall furnish, erect and maintain all signs; barricades shall be in place before any work is started by the contractor.

- In the event that a construction site in the opinion of the Department of Police, Right-of-Way Management, or the Transportation Engineering Division is improperly signed, barricaded, or lighted, the contractor will be so notified. If the contractor cannot be reached immediately or fails to take necessary corrective measures, a stop work order may be issued until such corrective action is taken.  Additional time for stop work order due to improper signage will not be granted.

- On streets with unlimited parking, No Parking signs shall be erected a minimum of 72 hours in advance of the beginning of any bridge work, street construction or street cleaning. Such signs shall bear the effective day and time. Signs prohibiting parking shall be placed not more than 100 feet apart. All existing regulatory signs which do not apply during construction shall be covered.  Parking Meters shall be covered with bags or temporary “No Parking” Signs approved by the Transportation Engineering Division.

- All construction signing, barricades and other traffic control devices shall be removed promptly when the construction is completed.

- The contractor shall maintain all signs and barricades in the roadway in the manner prescribed. All traffic control devices shall be kept in proper position, clean and legible at all times. Damaged traffic control devices shall be repaired or replaced immediately. Paper parking regulation signs shall be affixed so they will remain legible at all times, especially during inclement weather. In the event that work is halted, all unnecessary signs will be removed.

- Traffic signs, signals, or pavement markings called out to be relocated and removed as a result of the construction shall be replaced by the contractor.  Before removing any traffic control device, the Transportation Engineering Division shall be notified in order that temporary safety arrangements can be made. Any signs removed shall be delivered to the City’s Sign Shop by the person removing same or agent.

- Single Lane Traffic Control:

- Where traffic in both directions must use a single lane for a limited distance, provisions must be made for alternate one-way movement to pass vehicles through the constructed section.  At a “spot” obstruction, such as isolated pavement patch, the movement may be self-regulating.   However, where the one-lane movement section is of sufficient length, there must be means of coordinating movements at each end so that vehicles are not simultaneously moving in opposite directions in the section and delays are not excessive at either end. Control points at each end of the route must be chosen to permit easy passing of opposing lines of vehicles.

- Unless noted otherwise on the Maintence of Traffic Plan, the following minimum lane widths should be maintained at all times:

- One Lane  		10 Feet Wide

- Two Lanes			20 Feet Wide

- BIRD NETTING SYSTEM – The Contractor is responsible for providing and installation of bird netting system as per the Special Provision appendex 1.

- EROSION CONTROL AND TREE PROTECTION

      - DESCRIPTION

## The lump sum bid price shall include, but not limited to, installing and maintaining all erosion control measures and/or tree protection as shown on the Drawings. Placement of all erosion control measures shall be as indicated on the plans or as directed by the Owner's Representative due to field conditions. The lump sum bid price shall include all materials, labor, equipment and incidentals necessary to complete and maintain the work including the maintenance of items for which the provision and installation is paid for through another line item. The Contractor shall exercise reasonable precaution, including the application of temporary measures, throughout the duration of the project to control erosion and prevent or minimize siltation. Such measures shall include, but are not limited to, the use of silt fence, tree protection fences, construction entrances, temporary and permanent seeding, straw or silt barriers and other methods. Temporary measures shall be coordinated with the construction to assure economical, effective and continuous erosion and siltation control. All methods and materials shall be in accordance with the current Virginia Erosion and Sediment Control Handbook.

      - MEASUREMENT AND PAYMENT

## Item will not be measured separately but paid for on a lump sum basis. The work performed and materials furnished in accordance with this item and measured as provided under “Measurement” will be paid for at the Contract lump sum bid price for the successfully completed installation, maintenance, and removal (as applicable) of erosion control and tree protection as described in the plan. This price shall be full compensation for all manipulations, labor, equipment, materials, accessories, and incidentals necessary to complete the work. The lump sum price shall also include off-site removal/disposal of all unsuitable material in accordance with applicable local, state, and federal regulations and all other incidental items to complete this work. The cost of mulching and watering will be considered incidental to the Contract lump sum bid price for Erosion Control and Tree Protection with no separate payment thereto.

## Payment will be made under:

| ## Pay Item | ## Pay Unit |
| --- | --- |
| ## NS EROSION CONTROL AND TREE PROTECTION | ## LS |