Statement of Work

PR #1300531761

B148 Welding Duct Exhaust Systems Cleaning

This statement of work is for a one time cleaning service needed for the duct exhaust systems in B148 (Hangar 2) for NAVAIR 4.8.8 Prototype and Manufacturing Division. There are three separate ducting systems, along with their associated pick up points, hoods, and exhaust fans that will require interior and exterior cleaning. Site visits may be required to review job prior to quoting.

The follow services and tasks shall be provided by the vendor:

- Contractor shall provide all necessary cleaning and air quality technicians, materials, and equipment to complete the proposed project.

- Contractor shall use source removal methods for the complete removal of interior surface contaminates

- Cleaning shall be for the (3) Duct Exhaust Systems and ductwork, located in Building 148.

- Cleaning shall consist of all pick-up points, hoods, and the entire length of ductwork for the EF-14, EF-16, and EF-18.

- Access openings shall be installed in the exhaust duct for cleaning. Existing access doors shall be used and contractor can install new access doors where necessary.

- All work shall start at pick up flex duct and hoods. Any of these items found to have any accumulated dust cake shall be cleaned.

- Clean exhaust fans and ductwork.

- Clean dust on exterior of exhaust ductwork.

- Any access openings on hoods shall be painted flat black for easy identification.

- Contractor shall provide their own lifts, ladders, and scaffolds needed to properly complete this project.

- All debris collected shall be disposed of by contractor.

- All contractors shall wear the necessary personal protective equipment.

- All floors and areas immediately below the Exhaust Ductwork under maintenance shall be covered with protective cloths or equivalent to contain any fallen debris or dust.

- All ductwork shall have before and after photographs taken and provided to the Navy. All photographs taken need to be reviewed and approved by a government official before dissemination.

Robert M. Stangler

Industrial Specialist

Code 4881

Robert.stangler@navy.mil